Standard Operating Procedure

Preparing agar plates

Safety notes

- Maintenance: All equipment must be maintained according to manufacturer’s instructions.

- Handling: Attach warning signage to hot equipment. Follow good laboratory practice by limiting the handling of hot equipment and thoroughly washing hands after handling.

- Storage: Ensure the lid of the jar of purchased nutrient agar is tightly sealed before storing.

- Labelling: Write the preparation date in small numerals at the edge of the plate.

Equipment

- Personal protective equipment (disposable apron and safety glasses)

- Autoclave

- Laminate flow unit

- Balance

- Measuring cylinder

- Conical flask with non-absorbent cotton wool plug or Schott bottle sized to hold the required amount

- Hotplate stirrer and magnetic flea (stirring bar)

- Petri dishes: Sterile plastic Petri dishes, 90mm diameter are recommended for ease of use

- Disinfectant: 70 per cent ethanol for cleaning benches before and after pouring agar into plates

- Prepare 70 per cent ethanol by measuring 70 ml ethanol (or methylated spirits) and making up to 100 ml with distilled or deionised water. Note: 70 per cent Ethanol is flammable

- Nutrient agar (commercially available agar with premixed nutrients)

- Plain agar ingredients – agar and distilled water

St John Rigby College is delivering this programme on behalf of the Education and Training Foundation. This programme is funded by the Department for Education.

Operating procedure

To make 1 litre, sufficient for approximately 50–60 plates:

- Using a digital balance, prepare nutrient agar medium according to the manufacturer’s instructions.

- Place water in a 1 litre beaker. Using the hotplate stirrer and magnetic stirring bar, mix ingredients with the water whilst stirring to prevent clumping. Heat whilst stirring until the agar dissolves at 95°C. The agar is poured into conical flasks (autoclave safe). The lid of the bottle is to be closed fully, then released by a 1/4 turn prior to sterilising.

- Sterilise the conical flask autoclave 15–20 minutes at 121°C, 15 psi, according to manufacturer’s instructions for use of this equipment.

- The pressure in the vessel should be at zero prior to opening the equipment. The sterilised agar is allowed to cool to 50–55°C. This is the optimal temperature for pouring to minimise condensation. It might be useful to sit the flask or bottle in a water bath at this temperature to maintain the optimal temperature prior to pouring plates. Agar solidifies at 42°C.

- Thoroughly wash and dry hands, wear a disposable apron.

- Create a sterile area by thoroughly cleaning the laminate flow unit (LAF) with 70 per cent IPA solution according to standard operating procedure (SOP) on the operation, cleaning and maintenance of LAF.

- Spray sterile Petri discs with 70 per cent IPA solution and place in LAF. Label the sterile Petri dishes and organise them base down (the smaller section of the two halves) on the bench.

- Spray the flask containing sterile agar medium in liquid into the LAF.

- Remove the lid of the Petri dish. Pour approximately 15 to 20 ml of the sterilised nutrient agar medium into the base of the Petri dish, until it is about half full. Hold the Petri dish lid so that it partially covers the bottom of the dish as you pour.

- Immediately place the lid on the base at an angle so that steam is able to escape.

- Pour the remaining plates the same way.

- Allow the plates to cool to room temperature for approximately 30 minutes before packing upside down into labelled plastic bags, or use the bags the plates came in. Refrigerate the plates. Plates can be stored for several weeks prior to use.

St John Rigby College is delivering this programme on behalf of the Education and Training Foundation. This programme is funded by the Department for Education.

Waste disposal

- Cleaning up: All equipment used in the preparation of agar is to be washed thoroughly in warm soapy water, rinsed and dried.

- All agar plates, including plates not cultured, must be sterilised or autoclaved prior to being placed in double plastic garbage bag and sealed for immediate disposal in the industrial bins.

Related material

- Manufacturer’s safety data sheets

- Risk assessment

- Manufacturer’s instructions for autoclave

- SOP – Operation, cleaning and maintenance of LAF

- SOP – Operation, cleaning and maintenance of autoclave

| Version history notes |  |  |
| --- | --- | --- |
| Version 1.0 | 28.4.19 |  |
| Version 2.0 | 21.5.22 | Amended to include update to equipment with introduction of laminate flow unit into the laboratory |

St John Rigby College is delivering this programme on behalf of the Education and Training Foundation. This programme is funded by the Department for Education.