Standard Operating Procedure

Dilution streaking of bacterial culture

Safety notes

- Maintenance: All equipment must be maintained according to manufacturer’s instructions.

- Handling: Attach warning signage to hot equipment. Follow good laboratory practice by limiting the handling of hot equipment and thoroughly washing hands after handling.

- Storage: Ensure culture beads are stored at -10°C to -25°C for no longer than 12 months. Please note that cultures should be subcultured onto new slopes every 12 months in line with standard operating procedure (SOP) on maintaining bacterial cultures.

- Labelling: Write the preparation date, microorganism and operator details in small numerals at the edge of the plate.

Equipment

- Personal protective equipment

- Autoclave

- Sterile nutrient agar 90 mm diameter plates

- Disinfectant: 70 per cent ethanol for cleaning benches before and after pouring agar into plates

- Prepare 70 per cent ethanol by measuring 70 ml ethanol (or methylated spirits) and making up to 100 ml with distilled or deionised water. Note: 70 per cent ethanol is flammable

- Metal loop

- Freeze-dried culture beads

- Sterile pipette tips

- TSB (trypticase soya broth)

- Incubator

- Bunsen burner

Operating procedure

- Open a freeze-dried culture beads of the microorganism within sterilised laminar flow unit. Suspend on bead in 10ml TSB.

- Incubate the bacterial culture at 32.5C ± 2.5C for 48 hours.

- Create a sterile area by thoroughly wiping the bench with 70 per cent alcohol and paper towel. Do not completely dry the ethanol – allow it to evaporate. Air conditioners, fans and fume cupboards must be turned off, windows and doors closed, and foot traffic restricted to reduce the chance of contamination to plates while subculturing.

- Working close to a roaring Bunsen flame, put the plate the right way up.

- Flame an inoculation loop and allow it to cool while remaining close to the Bunsen flame.

- Using aseptic techniques, dip the loop into the culture to take a sample.

- Lift the edge of the plate closest to the flame and add the drop of culture from the loop to the starting circle.

- Re-flame the loop, taking care to avoid any aerosols being formed. Allow it to cool, again close to the flame.

- Using the loop, streak the culture from the drop along the first set of lines. Three to five streaks will be fine.

- Rotate the plate anticlockwise (clockwise for left-handed streaking) and streak the next set of lines.

- Keep rotating the plate and streaking until all the lines have been streaked.

- Flame the loop when you have finished.

- Incubate the bacterial culture at 32.5C ± 2.5C for 48 hours.

Waste disposal

- Area should be disinfected prior to and after activity.

- All agar plates and other biological waste must be autoclaved prior to being placed in double plastic garbage bag and sealed for immediate disposal in the industrial bins.

Related material

- Manufacturer’s safety data sheets

- Risk assessment

- Manufacturer’s instructions for autoclave

- SOP – Maintaining bacterial cultures

- SOP – Operation, cleaning and maintenance of incubator

- SOP – Preparing agar plates

- SOP – Operation, cleaning and maintenance of autoclave

| Version history notes |  |  |
| --- | --- | --- |
| Version 1.0 | 12.3.96 |  |
| Version 2.0 | 20.4.16 | Amended to include catch in culture storage from slopes to freeze-dried |

St John Rigby College is delivering this programme on behalf of the Education and Training Foundation. This programme is funded by the Department for Education.