Standard Operating Procedure

Subculturing bacteria and dilution streaking

Safety notes

- Maintenance: All equipment must be maintained according to manufacturer’s instructions.

- Handling: Attach warning signage to hot equipment. Follow good laboratory practice by limiting the handling of hot equipment and thoroughly washing hands after handling.

- Storage: Ensure samples are stored with fridge within temperature range 2 to 10ºC.

- Labelling: Write the preparation date, sample number, dilution factor and operator details in small numerals at the edge of the plate.

Equipment

- Personal protective equipment

- Stomacher

- Sterile nutrient agar 90 mm diameter plates

- Disinfectant: 70 per cent ethanol for cleaning benches before and after pouring agar into plates

- Prepare 70 per cent ethanol by measuring 70 ml ethanol (or methylated spirits) and making up to 100 ml with distilled or deionised water. Note: 70 per cent ethanol is flammable

- Sterile disposable plastic spreaders

- 1 ml micropipette (1,000 µl)

- Sterile pipette tips

Operating procedure

- Homogenise the food sample using a stomacher at a speed of 250 cycles/min for two minutes using 100 ml of maximum recovery dilution medium (0.1 per cent peptone, 0.85 per cent sodium chloride solution). The suspension contained in the stomacher bag is not a dilution.

- Create a sterile area by thoroughly wiping the bench with 70 per cent alcohol and paper towel. Do not completely dry the ethanol: allow it to evaporate. Air conditioners, fans and fume cupboards must be turned off, windows and doors closed, and foot traffic restricted to reduce the chance of contamination to plates while subculturing.

- Working aseptically, close to a roaring Bunsen flame, dilute this suspension to 10-8 by carrying out 10-fold steps using the maximum recovery dilution medium.

Note: In preparing the dilutions, 1 ml is transferred from the 100 suspension into 9 ml of diluent to give a dilution of 10-1. This procedure is repeated for further dilutions using a fresh sterile pipette for each decimal dilution. The purpose of the dilutions is to ensure that the colony count per plate is in the range of 15–300 cfu.

- Aseptically, working close to a roaring Bunsen flame, place 0.5 ml of 10-6 diluted suspension into the centre of an agar plate using a micropipette with a sterile removable tip. Discard the tip in disinfectant. Spread the solution evenly across the surface of the agar while keeping the lid covering the plate using a sterile disposable plastic spreader. Discard the spreader into the biological waste bin.

- Repeat step 4 for 10-7 and 10-8 dilutions.

- Allow the plates to dry, invert and incubate the at 32.5C ± 2.5C for 48 hours

- Count the colony forming units for the dilution which give the colony count per plate in the range of 15–300 cfu.

- Calculate the number of cfu per 100 ml sample suspension using formula below.

- CFU x 2 x dilution factor x 100.

- Record the result and batch number in laboratory records including the date of the examination. These laboratory records must be maintained for a period of not less than 18 months after the date of sampling.

- Compare the result to release standards for food type.

Waste disposal

All agar plates and other biological waste must be sterilised or autoclaved prior to being placed in double plastic garbage bag and sealed for immediate disposal in the industrial bins.

Related material

- Manufacturer’s safety data sheets

- Risk assessment

- Manufacturer’s instructions for autoclave

- Standard operating procedure (SOP) – Preparing agar plates

- SOP – Operation, cleaning and maintenance of incubator

- SOP – Operation, cleaning and maintenance of autoclave

| Version history notes |  |  |
| --- | --- | --- |
| Version 1.0 | 25.4.18 |  |
| Version 2.0 | 14.2.22 | Amended to include additional links to SOPs |
| Version 3.0 | 4.5.22 | Update to deposable spreaders from glass |

Reference material

https://assets.publishing.service.gov.uk/government/uploads/system/uploads/attachment_data/file/363146/Guidelines_for_assessing_the_microbiological_safety_of_ready-to-eat_foods_on_the_market.pdf

St John Rigby College is delivering this programme on behalf of the Education and Training Foundation. This programme is funded by the Department for Education.